TECHNICAL CHARACTERISTICS

Version:		1.1

Date:		02.02.2015

Name:			GREEN LIFT® - TML® 900KG THREE-IN

Short name:		GL TML THREE-IN

Characteristics:	hydraulic lift suitable for the transport of disable persons

Manufacturer:	GMV

Lift type:	GREEN LIFT - TML

Load:	900 kg

No of persons:	12

No of stops:	2-7

Travel:	max. 20 m

Cabin:	TML-type

dim. WxDxH	1400 x 1400 x 2170 mm;

no of entrances:	3

finishing	push button panel	stainless steel INOX

panels *		laminate

painted steel Polimod

stainless steel INOX

glass

floor: *			PVC

rubber

coconut

mirror: *		half a wall

full wall

lighting: *		fluorescent

LED

Doors:	GMV-Victory

dim. SxH	900 x 2000 mm

type: 			telescopic

finishing: *		painted steel Polimod

stainless steel INOX

glass

option: 		shaft doors

fire rated  class*	EI 30

EI 60

EI 90

Shaft dimensions:

pit:	1300 mm

headroom:	3400 mm

width:	2040 mm

depth:	1900 mm

Cabin speed: *	0,40 - 0,52 - 0,62 m/s

Drive type:	hydraulic / fluitronic

Drive ratio:	1 : 2

Tank: *	F1 / T2

Power: *	9,5 – 11,0 – 12,5 – 14,7 kW (depending on speed)

Valve block:	NGV proportional

Controller:	GMV-NEOS / microprocessor

Travelling mode: *	collective down

collective up/down

Machinery:	room

cabinet  F-type  –  dim. WxDxH ( 1000x650x2100 mm )

cabinet S1-type –  dim. WxDxH ( 1150x900x2150 mm )

Phone line: * 	PSTN / GSM (a GMV delivery)

Power supply:	400V / three-phase

Application:	new and existing buildings

*  -  select the right option

Other information in the drawings, brochures and on the www.gmv.pl website:

- TMC CABINS .pdf

or under:

e-mail:		proj@gmv.pl

phone:		+ 48 22 / 842 41 70

__________________________________________________________________________________________

GMV Polska Sp. z o.o.    02-954 Warszawa,  ul. Marconich 2 lok. 2, Poland

Phone +48 22 / 651 91 45,   842 41 70      Fax +48 22 /  858 99 69      Mobil.   +48 601  27 39 22      E-mail: info@gmv.pl