TECHNICAL CHARACTERISTICS

Version:		1.0

Date:		15.02.2018

Name:			GREEN LIFT® GLT

Short name:		GLT

Characteristics:	traction lift suitable for the transport of disable persons

Manufacturer:	GMV

Lift type:	GLT MRL 630 kg

Load:	630 kg

No of persons:	8

No of stops:	2-12

Travel:	max. 35 m

Cabin:	TMC-type

dim. WxDxH	1100 x 1400 x 2170 mm;

no of entrances:	1-opening

2-opening

finishing	structure: *		painted steel Polimod / grey color

stainless steel INOX

panels *		laminate

painted steel Polimod

stainless steel INOX

glass

floor: *			PVC

rubber

coconut

mirror: *		half a wall

full wall

lighting: *		fluorescent

LED

Doors:	GMV-Victory

dim. SxH	900 x 2000 mm

type:			telescopic

central

finishing: *		painted steel Polimod

stainless steel INOX

glass

option: 		shaft doors

fire rated  class*	EI 30

EI 60

EI 90

Shaft dimensions:

pit:	1000 mm

headroom:	3500 mm

width:	1600 mm (telescopic doors)

1950 mm (central doors)

for 1-opening cabin

depth: 	1750 mm (telescopic doors)

1700 mm (central doors)

for 2-opening cabin

depth:	1940 mm (telescopic doors)

1880 mm (central doors)

Cabin speed: *	1,0 m/s

Drive type:	belt-gearless

drive ratio:	2 : 1

Power: *	5,5 kW

Controller:	GMV-NEOS / microprocessor

Control cabinet	integral with the shaft

dim. SxGxH:	440 x 270 x 2210 mm

Travelling mode: *	collective down

collective up/down

Machinery: *	no

Phone line: *	PSTN / GSM (a GMV delivery)

Power supply:	400V / three-phase

Application:	new and existing buildings

*  -  select the right option

Other information in the drawings, brochures and on the www.gmv.pl website:

- GREEN LIFT GLT .pdf

- TMC CABINS .pdf

or under:

e-mail:		proj@gmv.pl

phone:		22 / 842 41 70

__________________________________________________________________________________________

GMV Polska Sp. z o.o.    02-954 Warszawa,  ul. Marconich 2 lok. 2, Poland

Phone +48 22 / 651 91 45,   842 41 70      Fax +48 22 /  858 99 69      Mobil.   +48 601  27 39 22      E-mail: info@gmv.pl

__________________________________________________________________________________________

GMV Polska Sp. z o.o.    02-954 Warszawa,  ul. Marconich 2 lok. 2, Poland

Phone +48 22 / 651 91 45,   842 41 70      Fax +48 22 /  858 99 69      Mobil.   +48 601  27 39 22      E-mail: info@gmv.pl