TECHNICAL CHARACTERISTICS

Version:		1.0

Date:		12.02.2015

Name:			GOODS-PASSENGER LIFTS GPL®

Short name:		GPL®

Characteristics:	hydraulic goods-passenger lifts adapted for transportation of goods

and persons

Manufacturer:	GMV

Lift type:	1.) GPL 900-1200 MRL-MC

2.) GPL 1000-1600 MRL-MC

Load:	1.) 900 - 1200 kg

2.) 1000 – 1600 kg

No of persons:	1.) 12

2.) 21

No of stops:	2-8

Travel:	max. 20 m

Cabin:	GMV-GPL-type

dim. WxDxH	1.) 1400 x 1500 x 2170 mm (dla GPL 900-1200)

2.) 1400 x 2400 x 2170 mm (dla GPL 1000-1600)

no of entrances:	1 (1 entrance) / 2 (2 entrances)

finishing	panels: *		      painted steel Polimod

stainless steel INOX

floor: *			      painted steel grooved

stainless steel INOX

lighting: *		      fluorescent

LED

option:	bumpers on the walls: *  wooden

steel

Doors:	GMV-Victory

dim. SxH	1.) 1000 x 2000 mm

2.) 1200 x 2000 mm

type: *			      telescopic

central

finishing: *		      painted steel Polimod

stainless steel INOX

glass

option:	bumpers on the walls *   wood

steel

shaft doors fire rated  class *	      EI 30

EI 60

EI 90

sils *	aluminum

steel

steel reinforced

Shaft dimensions:

pit:	1300 mm

headroom:	3400 mm

width and depth:	according to the drawing GPL 900-1600

Cabin speed: *	0,30 – 0,40 - 0,52

Drive type:	hydraulic / fluitronic

Power unit:	F1 / T2 / T3

Motor power: *	1.) 7,7 - 9,5 - 11,0 (depending on the speed)

2.) 12,5 – 18,4 - 22,0 (depending on the speed)

Valve block:	3010 DLV / NGV

Controller:	GMV-NEOS / SEA - microprocessor

Travelling mode:	selective

Option:	door closing suspension

Machinery:	room

cabinet F type – dim.  SxGxH ( 1000x650x2100 mm )

cabinet S1 type – dim. SxGxH ( 1150x900x2150 mm )

Phone line: *	PSTN / GSM (a GMV delivery)

Power supply:	400V / three-phase

Application:	new and existing buildings

*  -  select the right option

Other information in the drawings, brochures and on the www.gmv.pl website:

- GPL Goods-passenger lifts .pdf

or under:

e-mail:		proj@gmv.pl

phone:		+ 48 22 / 842 41 70

GPL® Goods-Passenger Lifts by GMV - the most perfect lift devices 2.000-12.500 kg that have ever been invented

__________________________________________________________________________________________

GMV Polska Sp. z o.o.    02-954 Warszawa,  ul. Marconich 2 lok. 2, Poland

Phone +48 22 / 651 91 45,   842 41 70      Fax +48 22 /  858 99 69      Mobil.   +48 601  27 39 22      E-mail: info@gmv.pl

GPL® Goods-Passenger Lifts by GMV - the most perfect lift devices 2.000-12.500 kg that have ever been invented

__________________________________________________________________________________________

GMV Polska Sp. z o.o.    02-954 Warszawa,  ul. Marconich 2 lok. 2, Poland

Phone +48 22 / 651 91 45,   842 41 70      Fax +48 22 /  858 99 69      Mobil.   +48 601  27 39 22      E-mail: info@gmv.pl