TECHNICAL CHARACTERISTICS

Version:		1.1

Date:		26.10.2015

Name:			HOME LIFT® 2 ENTRANCES

Short name:		HL – MC – 2W

Characteristics:	hydraulic lift in accordance with the Machinery Directive 2006/42/EC

Manufacturer:	GMV

Lift type:	HOME LIFT® 2 ENTRANCES

Load:	250-400 kg

No of persons:	3 / 5

No of stops:	5

Travel:	max. 14 m

Cabin:	HL-type

dim. WxDxH	1100 x 1400 x 2170 mm	(version   I  – 400 kg)

900 x 1200 x 2170 mm	(version  II – 350 kg)

800 x 1200 x 2170 mm	(version III – 350 kg)

800 x 1000 x 2170 mm	(version IV – 250 kg)

no of entrances:	2 (2 entrances)

finishing	structure: *		painted steel Polimod / grey color

stainless steel INOX

panels *		laminate

painted steel Polimod

stainless steel INOX

glass

floor: *			PVC

rubber

coconut

mirror: *		half a wall

full wall

lighting: *		fluorescent

LED

Doors:	GMV-Victory

dim. SxH	900 x 2000 mm	(version   I)

800 x 2000 mm	(version  II)

750 x 2000 mm	(version III)

700 x 2000 mm	(version IV)

type: *			telescopic

central

finishing: *		painted steel Polimod

stainless steel INOX

glass

option: 		shaft doors

fire rated  class*	EI 30

EI 60

EI 90

Shaft dimensions:

pit:	150 mm

headroom:	2600 mm

width and depth:	according to the drawing HOME_LIFT_250-400_kg_2_entrances

Cabin speed: *	0,15 m/s

Drive type:	hydraulic / fluitronic

drive ratio:	1 : 2

Power unit:	HL DRY

Motor power: *	1,5 – 2,2 kW (depending on load)

Valve block:	NGV proportional

Controller:	GMV-NEOS / microprocessor

Travelling mode: *	APB

Machinery:	room

cabinet HL type – dim. SxGxH ( 650x350x1800 mm )

Phone line: *	PSTN / GSM (a GMV delivery)

Power supply:	230V / one-phase

Application:	new and existing buildings

*  -  select the right option

Other information in the drawings, brochures and on the www.gmv.pl website:

- HOME LIFT GMV .pdf

or under:

e-mail:		proj@gmv.pl

phone:		+ 48 22 / 842 41 70

__________________________________________________________________________________________

GMV Polska Sp. z o.o.    02-954 Warszawa,  ul. Marconich 2 lok. 2, Poland

Phone +48 22 / 651 91 45,   842 41 70      Fax +48 22 /  858 99 69      Mobil.   +48 601  27 39 22      E-mail: info@gmv.pl