TECHNICAL CHARACTERISTICS

Version:		1.2

Date:		17.02.2016

Name:			GMV VEHICLE LIFT 3500 / 3000 KG

Short name: *		VL® 35  /  VL® 30 /  VL® 35 H  /  VL® 30 H (H option)

Characteristics:	hydraulic vehicle lifts

Manufacturer:	GMV

Lift type:	VL®

Load:	3500 kg / 3000 kg

Force on sill Fs	2100 kG / 1800 kG

No of persons:	number of seats in a car

No of stops:	12

Travel:	max. 20 m

Cabin: *	VL –type / VL –type H

dim. WxDxH *	2700 x 5600 x 2200 mm / 2700 x 5600 x 2500 mm   (load 3500 kg)

2400 x 5300 x 2200 mm / 2400 x 5300 x 2500 mm   (load 3000 kg)

no of entrances:	1-opening

2 -opening

finishing	panels *		painted steel Polimod

stainless steel INOX

floor: *			painted steel						      			      	stainless steel

lighting: *		fluorescent

LED

Doors:

dim. SxH *	2400 x 2000 mm / 2400 x 2500 mm (H)	(load 3500 kg)

2300 x 2000 mm / 2300 x 2500 mm (H)	(load 3000 kg)

type: *			central 6-panel

finishing: *		painted steel Polimod

stainless steel INOX

option:		shaft doors fire rated  class: *     EI 60

EI 90

Shaft dimensions:

pit:	1000 mm

headroom:	according to the GMV guidelines

width: 		3400 mm 	(load 3500 kg)

3300 mm 	(load 3000 kg)

depth:	for load 3500 kg

6100 mm (1-opening)

6340 mm (2-opening)

for load 3000 kg

5800 mm (1-opening)

6040 mm (2-opening)

Cabin speed: *	0, 30 m/s

Drive type:	hydraulic

drive ratio:	1 : 1 (GMV technology)

Power unit:	T4 / 2xT3

Motor power: *	29,4 kW

Valve block:	3010 DLV / NGV

Controller:	GMV-NEOS / SEA - microprocessor

Travelling mode: *	selective (remote control of lift recalling)

Machinery:	room

Phone line: *	PSTN / GSM (a GMV delivery)

Power supply:	400V / three-phase

Application:	new and existing buildings

*  -  select the right option

Other information in the drawings, brochures and on the www.gmv.pl website:

- VL VEHICLE LIFT .pdf

or under:

e-mail:		proj@gmv.pl

phone:		+ 48 22 / 842 41 70

VL® Vehicle Lifts by GMV – the most perfect lift devices to carry vehicle that have ever been invented

__________________________________________________________________________________________

GMV Polska Sp. z o.o.    02-954 Warszawa,  ul. Marconich 2 lok. 2, Poland

Phone +48 22 / 651 91 45,   842 41 70      Fax +48 22 /  858 99 69      Mobil.   +48 601  27 39 22      E-mail: info@gmv.pl